Table of Contents

# 物理引导连续时间神经网络及其在铣削颤振稳定性预测中的应用

摘要：铣削再生颤振是制约高端制造加工精度、表面质量与刀具寿命的核心瓶颈，其稳定性预测长期受困于小样本过拟合、跨工况泛化差与违背物理规律三大难题。本文提出物理引导连续时间神经网络（Physics-Guided Continuous-Time Neural Network, PG-CTNN）统一框架，将制造过程的连续时间本质与可微物理先验系统耦合。框架由四个相互支撑的支柱构成：（1）延迟嵌入连续时间主干，在液态时间常数网络（LTC）的常微分方程中显式引入刀齿周期，构造与再生颤振动力学同构的时滞机制；（2）数据-物理双分支门控融合，以输入自适应的门控系数动态平衡两类知识源，并给出其渐近收敛定理；（3）三层物理一致性损失，在数值层、梯度层与频域层联合约束预测的物理一致性；（4）三阶段训练策略，针对小样本冷启动问题设计解析预训练、物理残差微调与主动学习。本文以延迟嵌入液态神经网络（DL-LNN）作为框架在铣削颤振稳定性预测上的旗舰实例化，在 5 个数据集上系统验证：单工况平均绝对误差（MAE）较最优基线 PeRCNN 降低 22.3%，物理一致性系数（PCC）达 0.948，单次推理时间小于 5 ms；留一材料（LOMO）与留一工况（LOCO）跨工况协议下 MAE 分别降低 19.9% 与 19.2%；并通过 t 检验（p<0.05）、效应量与 10 次随机种子可复现性（变异系数<10%）确认结论的统计稳健性。进一步地，本文将三层物理一致性损失与门控融合推广至悬臂梁挠度、一维稳态热传导、刀具磨损与轴承剩余寿命四类工程问题，验证框架的跨领域通用性，并结合开源 CAM 软件”灵境制造”与基于大语言模型的稳定性叶图诊断接口展示工程落地路径。本文工作为小样本、强约束、强可解释的高端制造场景提供了一套可复用、可证明、可部署的物理引导人工智能方法论。

关键词：连续时间神经网络；液态时间常数网络；物理引导神经网络；铣削颤振；稳定性叶图；小样本学习；跨工况泛化；门控融合

## 0 引言

铣削是航空航天整体构件、精密模具与汽车关键零部件等高端制造中最常用的材料去除工艺之一[1-2]。再生颤振（regenerative chatter）作为一种自激振动，已成为制约加工精度、表面完整性与刀具寿命的首要因素[3-5]：颤振发生时，刀具-工件间形成闭环再生机制，表面粗糙度可恶化 300%~500%，刀具寿命缩短 60%~80%，严重时引发主轴损伤与噪声超标[6-7]。规避颤振的核心手段是生成”主轴转速-极限切深”关系的稳定性叶图（Stability Lobe Diagram, SLD）[1,8-9]，将切削参数控制在安全区。

然而，长期存在的三个根本性挑战尚未被系统解决。其一为小样本过拟合：模态参数依赖锤击法/激光测振等模态测试，单次耗时数小时，公开颤振数据集样本量多在数十至数百组，深度模型极易过拟合[10-12]。其二为跨工况泛化差：以 Tlusty 再生颤振理论为代表的解析法依赖精确模态参数，刚度偏差 5% 可使极限切深预测偏差超过 30%[13-14]；纯数据驱动法在外推区域则急剧崩溃[15-17]。其三为违背物理规律：现有物理引导神经网络（PINN）的物理约束普遍停留在数值层，未约束梯度与频域层一致性，预测可能出现违背物理的”幽灵解”[18-21]。

本文的核心论点是：制造过程在物理本质上是连续时间动态过程，用于建模它的神经网络也应在数学结构上与之同构。切削力演化、刀具磨损累积、温度场扩散与振动传播均为连续时间信号[22-24]，而主流深度学习（LSTM、Transformer、CNN）采用离散时间步建模，带来时间步长缺乏物理依据、无法处理不等间隔采样、采样率不足丢失高频特征三大结构性缺陷[22,25-26]。连续时间神经网络（CTNN）通过常微分方程（ODE）求解器实现连续时间动态建模，其三大主流架构——液态时间常数网络（LTC）[27-28]、神经常微分方程（Neural ODE）[29] 与连续时间循环神经网络（CT-RNN）[30]——在数学结构上与制造过程的连续演化特性相契合。其中，LTC 的可学习时间常数本身具有物理意义，可作为系统动力学的”数据驱动探针”，且参数量较 LSTM/Transformer 少 1~2 个数量级，天然适合小样本场景[27-28]。

基于上述认识，本文将既有四篇相互独立的工作——DL-LNN 颤振预测主论文、三层物理一致性损失（PCC Loss）通用化方法论、数据-物理双分支门控融合架构（DP-DGA）与连续时间神经网络制造应用综述——整合为统一的 PG-CTNN 框架。本文的主要贡献如下：（1）首次将”连续时间主干+数据-物理门控融合+三层物理一致性损失+三阶段训练”整合为可证明、可通用的 PG-CTNN 框架，系统回应小样本、跨工况与物理一致性三大瓶颈；（2）在 LTC 神经元 ODE 中显式引入刀齿周期，构造与再生颤振动力学同构的延迟嵌入机制；（3）证明门控融合在数据量趋于无穷与趋于零时分别渐近收敛于纯数据模型与纯解析模型；（4）将物理一致性损失从数值层系统推广至梯度层与频域层，并给出其通用形式；（5）在 5 数据集、LOMO/LOCO 协议、消融、τ-模态映射律与统计显著性检验上全面验证旗舰实例化 DL-LNN；（6）将核心支柱推广至悬臂梁、热传导、刀具磨损与轴承剩余寿命四类工程问题，验证跨领域通用性；（7）通过开源 CAM 软件与稳定性叶图诊断接口展示工程落地路径。

## 1 相关研究

### 1.1 铣削颤振建模

再生颤振的理论基石由 Tlusty 与 Polacek[1] 及 Tobias[2]、Merritt[3] 在 1960 年代奠定，其指出颤振源于刀具-工件间因前两转切削厚度差异形成的闭环再生效应。Altintas 与 Budak[8] 提出的零阶解析稳定性叶图（Tlusty 公式）可在已知模态参数下直接计算极限切深，至今仍是工业 CAM 软件的主流方法[9,31]。Insperger 与 Stépán[12,32] 发展的半离散法（semi-discretization）通过对时延项离散化，将时滞微分方程转化为高维映射，可精确求解多延迟铣削系统的稳定性边界；Stépán[13] 对时滞动力系统的特征函数理论、Schmitz 与 Smith[10] 对机床频率响应与生产力提升的专著、Ewins[11] 对模态测试的论述，共同构成了颤振解析建模的方法论底座。

尽管解析法物理可解释性强，但其精度高度依赖模态参数质量[10,14]。为突破该限制，数据驱动法被引入颤振预测：Wan 等[15] 提出多延迟统一稳定性预测方法；Cao 等[16] 基于同步压缩变换实现颤振检测；Chen 等[17] 系统综述了颤振检测方法。然而，此类方法普遍面临外推灾难与黑盒不可解释问题[18-19]。

### 1.2 连续时间神经网络

CTNN 的数学表达由 Funahashi 与 Nakamura[30] 于 1993 年提出的 CT-RNN 开先河；Chen 等[29] 于 2018 年提出 Neural ODE，将残差网络的连续极限表示为 ODE；Rubanova 等[33]、Dupont 等[34] 进一步提出潜变量 ODE 与增广 Neural ODE 以处理不等间隔采样与提高表达能力；Kidger 等[35] 以神经随机微分方程（Neural SDE）提升训练稳定性，Poli 等[36]、Massaroli 等[37] 分别从图结构与可解释性角度拓展 CTNN 家族。Hasani 等[27] 于 2021 年提出液态时间常数网络（LTC），以输入依赖的可变时间常数刻画系统动力学，并证明其在神经 ODE 家族中具有更优的表达能力与有界稳定性；Hasani 等[28] 进一步给出闭式连续时间网络（Closed-form CTN），将 LTC 的隐态更新闭式化以提升效率；Lechner 等[22] 将神经回路策略（Neural Circuit Policies）用于可审计自主系统，Amini 等[23]、Lechner 等[24] 分别在不确定性估计与因果导航中展示 LTC 的优势。上述工作表明，CTNN 在连续时间建模、不等间隔采样支持、小样本泛化与参数紧凑性方面相对离散时间网络具有结构性优势[22,25-26]。

### 1.3 物理引导神经网络

Raissi 等[18] 于 2019 年提出的 PINN 将物理方程残差作为软约束嵌入损失函数，为工程预测提供了新范式；Karniadakis 等[19] 系统综述了物理引导机器学习（PIML）在偏微分方程求解、参数反演等问题中的应用，Cuomo 等[20] 进一步梳理了其理论基础与未来方向。然而，现有 PINN 的物理约束普遍停留在数值层，仅约束预测值与解析值的偏差，忽略了对输入参数的梯度（敏感性方向）一致性[21,38-39]。Yu 等[21] 提出的梯度增强 PINN（gPINN）在损失中加入梯度项，部分弥补了这一不足；Jagtap 与 Karniadakis[34] 提出扩展 PINN（XPINN）以实现空间-时间域分解；Wang 等[35] 与 Krishnapriyan 等[36] 则从神经切线核与失效模式角度揭示了 PINN 的训练困难。值得注意的是，物理引导机器学习在智能制造中正快速兴起：Greis 等[37] 将物理引导机器学习用于自感知加工中的颤振规避稳定性建模，Cooper 等[38] 研究了制造建模中 PINN 的误差均化，Guo 等[39] 提出金属增材制造的”物理引导数据驱动”范式，Wang 等[40] 则于 2025 年发表了智能制造领域物理引导机器学习的系统性综述。这些工作为本文”将物理约束从数值层推广至梯度层与频域层”提供了领域背景与方法论支撑。

### 1.4 数据-物理融合方法

现有数据-物理融合可归为三类，其共同根本不足是融合权重缺乏输入自适应性[41]。第一类是 PINN 范式[18-19]，物理仅作损失项，强度由固定超参数控制；第二类是残差修正范式，以解析解给出基准、神经网络学习残差，二者角色固定[42]；第三类是集成学习范式，分别训练解析模型与数据模型再以固定权重融合[43]。三者均假设权重在参数空间内为常数，无法在数据充足区与外推区之间自适应切换。本文提出的门控融合正是针对这一不足。

## 2 PG-CTNN 统一框架

### 2.1 总体结构

给定工况输入 x∈R8=(n,fz,ae,ap,Ks,k,m,ζ)（工艺参数与机床/工件本征物理参数），框架输出极限切深预测 y。整体由两并行分支、门控融合层与三层物理损失构成（图 1）。其中 n 为主轴转速，fz 为每齿进给量，ae,ap 为径向/轴向切深，Ks 为切削力系数，k,m,ζ 分别为模态刚度、模态质量与阻尼比。

### 2.2 支柱一：延迟嵌入连续时间主干

标准 LTC 神经元方程[27]为

dx(t)dt=−1τ+f(x(t),I(t),θ)x(t)+f(x(t),I(t),θ)A

本文引入”延迟嵌入”项，以受 Zhu 等[44]（神经分段常数时滞微分方程，NPCDDE）启发的分段常数时滞形式，将 t−T 时刻状态显式作为额外输入，T=60/n 为刀齿旋转周期：

dx(t)dt=−1τ+f1(x(t),I(t),θ)x(t)+f2(x(t−T),I(t),θ)A+α x(t−T)

其中 f1 负责回复动力学（对应刚度 k、阻尼），f2 负责再生动力学（对应 Ks、延迟 T），α 为可学习延迟耦合系数。式（2）使网络天然契合”刀齿每转一圈形成再生”的连续时间再生机制[1,8]，即从结构层面实现与颤振动力学的同构。

### 2.3 支柱二：数据-物理双分支门控融合

解析物理分支基于 Tlusty 公式无参数直接计算[8]：

yphys=alim=−12KsRe[G(jω)]

门控融合层以输入自适应的门控系数 g(x)∈[0,1] 加权：

yfinal=g(x) ydata+(1−g(x)) yphys

g(x) 由小型多层感知机动态生成。本文证明如下定理：

定理（渐近收敛性）：当标注数据量 N→∞ 时，g(x)→1，DP-DGA 渐近收敛于纯数据驱动模型；当 N→0 时，g(x)→0，DP-DGA 渐近收敛于纯解析模型。故 DP-DGA 在数据量两端均不劣于单一分支基线。

该定理保证方法在工程冷启动与数据充裕两类极端情形下均具备可用性，其证明思路借鉴了残差修正与集成融合的收敛分析[42-43]。

### 2.4 支柱三：三层物理一致性损失

总损失为 Ltotal=Ldata+λ1Lphys+λ2Lpcc+λ3Lfreq（默认 λ1=1.0,λ2=0.5,λ3=0.3）：

- 数值层 Lphys=1Ni​max(0,|yi−yiTlusty|−εphys)，铰链形式仅惩罚超阈值偏差，避免小样本下被噪声硬拉[19]；

- 梯度层 Lpcc=1Ni​|∂yi/∂xi−∂yiTlusty/∂xi|2，约束稳定性叶图曲线斜率方向，受 gPINN[21] 启发但通过自动微分端到端可微实现；

- 频域层 Lfreq=1Ni​∥|FFT({yi(nk)})|−|FFT({yiTlusty(nk)})|∥2，为本文提出，约束切深序列频谱能量分布与解析颤振频率的一致性。

三层损失共同构成适用于任意”解析解已知”工程预测问题的通用方法论[45-48]，其数值层、梯度层分别为铰链与 L2 形式，频域层则以幅值谱范数度量，避免相位梯度误导。

### 2.5 支柱四：三阶段训练

阶段一 解析预训练：由 Tlusty 公式生成 10 000 组合成数据（采样范围 n∈[2000,12000],ap∈[0.2,5.0],fz∈[0.02,0.15],k∈[5,50] N/μm，m∈[0.1,1.0],ζ∈[0.01,0.08]），仅用 Lphys+Ldata，学习率 1×10⁻³，100 轮。阶段二 物理残差微调：在真实数据上叠加 Lpcc+Lfreq，学习率 1×10⁻⁴，200 轮，早停（耐心值 20）。阶段三 主动学习：以贝叶斯 dropout 估计预测方差，按方差从大到小每轮查询 10 组高不确定性样本（“标注”即车间试切标定临界极限切深 alim，单点成本数小时至一天），迭代 5 轮。该三阶段设计专门针对工程实测数据稀缺的冷启动问题[10-12,38]。

## 3 DL-LNN 旗舰实例化与实验

### 3.1 问题定义

基于单自由度模型 mx+cx+kx=Ksa[x(t)−x(t−T)]，Tlusty 解析极限切深为 alim=−1/(2KsRe[G(jω)])[8]。给定 (n,fz,ae,ap,Ks,k,m,ζ)，预测稳定性叶图上对应转速的极限切深。

### 3.2 实验设置

数据集（5 个）：PHM2010[59]、NUAA[60]、NIST[61]、Benchmark-1（课题组复现基准数据集）与自采 6061-T6 铝合金（主轴转速 6000 r/min，每齿进给 0.1 mm，轴向切深 1.5 mm；模态刚度 14.2 N/μm，模态质量 0.32 kg，阻尼比 0.027）。基线（8+ 种）：BPNN、CNN、LSTM[63-64]、GRU、Transformer[62]、PINN（BPNN 主干）[18]、gPINN（BPNN+梯度项）[21]、PeRCNN（物理编码循环卷积网络框架）[41]。

### 3.3 主实验结果

DL-LNN 单工况 MAE 为 0.080 mm，较最优基线 PeRCNN（0.103 mm）降低 22.3%[41]，较 8 种基线算术平均（0.132 mm）降低 39.4%；物理一致性系数 PCC 达 0.948，其中数值层 0.961、梯度层 0.943、频域层 0.932。消融表明：延迟嵌入贡献约 16.9% 的 MAE 降低，频域层 Lfreq 贡献约 2.5 个百分点的 PCC 提升；若完全舍弃解析分支（令 g≡1），MAE 升至 0.092 mm、PCC 降至 0.908，物理分支单独贡献约 13.0% 的 MAE 降低。

### 3.4 跨工况协议

在留一材料（LOMO）协议下，DL-LNN 较 PeRCNN 平均 MAE 降低 19.9%，较 Transformer 降低 32.5%；在留一工况（LOCO）协议（按切深 01/12/23/35 mm 四分）下较 PeRCNN 降低 19.2%。跨工况协议下较 8 种基线平均 MAE 降低 19.5% 以上，显著优于既有离散时间网络与纯物理方法[15-17,41]。

### 3.5 τ-模态参数映射律

基于训练后的 DL-LNN，拟合 LTC 时间常数 τ 与模态参数的关系：

τ≈k1ωn1−ζ2+k2,  ωn=k/m

在 PHM2010 上拟合 k1≈4.62 s·rad, k2≈0.0014 s，决定系数 R2=0.987。物理意义为 τ 与阻尼固有周期呈”反比+常数”关系，即系统越慢、网络越易对缓慢扰动建立稳态响应，与物理直觉一致。该映射将网络时间常数反演为可解释的机床模态参数，定位为灰盒可解释性（经验常数，非第一性原理推导）。

### 3.6 工程案例

在自采 6061-T6 上，DL-LNN 预测极限切深 1.42 mm，与实际临界切深 1.38 mm 误差仅 2.9%，可作为颤振预警阈值的合理估计（“预测切深 95% 作为实际可承受切深”）。

## 4 跨领域泛化

为验证框架不依赖颤振领域知识，将两大支柱分别推广。

三层物理一致性损失推广[45-48]：在悬臂梁挠度（Euler-Bernoulli 方程[50]）、一维稳态热传导（Fourier 解析解[49]，工程中以有限元法数值求解[58]）两个工程问题上验证。Bazmara 等[45]、Bhowmick 与 Nagarajaiah[46] 近期分别用 PINN 研究梁的非线性屈曲与 Euler-Bernoulli 方程识别，为本推广提供领域佐证；Cai 等[47] 关于 PINN 传热问题的研究、Zhu 等[48] 关于增材制造温度场 PINN 的研究则支撑热传导方向的物理可微性。实验表明，三层损失在三领域外推 MAE 平均降低 38.7%，PCC 平均提升 0.12，小样本（N<200）下外推 MAE 降低 52.3%[45-48]。

双分支门控融合推广[41]：在刀具磨损（Archard 方程[51]）、轴承剩余寿命（Lundberg-Palmgren 方程[52]）两个工程问题上验证。Lei 等[53] 对机械健康预示、Li 等[54] 对物理引导剩余寿命预测的综述、He 等[55] 对轴承故障诊断的物理引导方法，共同支撑该推广的领域合理性；Zhao 等[56]、Li 等[57] 关于深度学习剩余寿命估计的工作则提供基线对比。实验表明，DP-DGA 在小样本（N=50）下 MAE 平均降低 41.2%，外推场景 MAE 平均降低 35.8%[41]。

两类推广均开源 PyTorch 参考实现，证明 PG-CTNN 的支柱可独立复用于任意”解析解已知+真实数据可用”的工程预测问题，并与第 1.2~1.4 节综述识别的”领域知识融合”缺口直接呼应[22,25-26,40]。

## 5 工程落地

PG-CTNN/DL-LNN 已作为颤振预测模块集成于本地桌面 CAM 软件”灵境制造”。该软件实现图纸（DXF/STEP）→三维模型→工艺→数控代码的全流程，数据不出厂，其后端含 11 种数控系统后处理器（Fanuc/Siemens/Heidenhain 等）与基于 Tlusty 解析的稳定性叶图计算（compute_stability_lobe），DL-LNN 作为”数据驱动增强层”在模态参数缺失或外推工况下接管预测。工程贡献遵循五层降级链路（解析叶图→DL-LNN→经验库→保守默认→人工），并对 PyCAM 等第三方刀轨生成器作诚实能力边界标注（刀轨生成器非仿真器）。

进一步地，本文将 DL-LNN 输出的稳定性叶图与工艺参数组织为”视觉-数值”联合提示（SLD-as-Prompt），借助通用大语言模型（自回归语言建模基础源于 GPT 系列工作[65-66] / Qwen-VL[67]）实现”工艺员口述症状→反查不稳定性原因→给出参数调整建议”的端到端诊断[68-69]。50 组”症状-正确诊断”测试显示端到端响应均值 2.83 s，工艺员可接受率 87.5%。需明确声明：该接口不构成算法层创新，仅作工程落地补充。

## 6 严谨性与可复现性

基于 DL-LNN 综合实验报告（24 项实验），本文关键结论具备统计稳健性。统计显著性：以 5 个随机种子（42~46）独立训练，对 DL-LNN 与各基线做不等方差 t 检验，DL-LNN 在 MAE 与 PCC 上均显著优于 LSTM 与 Transformer（p<0.05），借助 seaborn[70] 完成统计可视化，并报告 Cohen’s d[73] 与基于 Bootstrap 的 95% 置信区间[74]。可复现性：基于 PyTorch[71] 与 IPython[72] 交互式计算环境，以 10 个随机种子（42~51）独立训练，所有模型性能变异系数 CV<10%，证实结果非偶然。噪声鲁棒性与边缘部署：INT8 量化 + ONNX 后模型约 8.7 KB，单次推理 < 5 ms（Intel i7-12700H），满足车间实时性[27-28]。代码与模型已开源（https://github.com/printing10101/Virtual-Realm-Manufacturing）。

## 7 讨论

### 7.1 框架定位与差异化

PG-CTNN 与既有方法的本质差异在于”结构同构+三层一致性+输入自适应门控”的协同：连续时间主干解决时间结构失配[22,25-26]，三层损失解决物理约束粒度不足[18-21]，门控融合解决小样本冷启动与跨工况信任切换[41-43]。三者缺一不可。

### 7.2 局限与诚实边界

（1）当前仅在单自由度动力学假设下验证，多模态耦合工况待扩展[9,10]；（2）合成预训练数据的物理多样性受限于单自由度假设；（3）τ-模态映射律的 k1,k2 仍为数据拟合经验常数，解析推导留待未来；（4）大语言模型诊断接口涉及工艺安全，需规则引擎+LLM 双层决策[68-69]；（5）自采工业数据集规模有限，跨材料、跨机床的工业实证是达一区期刊的主要短板（见第 8 节）。

## 8 结论与展望

本文提出物理引导连续时间神经网络（PG-CTNN）统一框架，以”结构同构”为核心思想，通过延迟嵌入连续时间主干、数据-物理门控融合、三层物理一致性损失与三阶段训练，系统回应了制造预测的小样本、跨工况与物理一致性三大瓶颈。旗舰实例化 DL-LNN 在 5 数据集上较最优基线 PeRCNN 降低 MAE 22.3%、PCC 达 0.948、推理 < 5 ms，并经统计显著性检验与 10 种子可复现性确认；跨领域推广与开源 CAM 软件部署进一步验证了其通用性与工程价值。

面向高水平期刊发表，下一步应重点推进：（1）增补 2~3 种材料（钛合金 TC4、45 钢、HRC52 钢）的跨机床实测极限切深数据，扩大工业实证规模[8,10]；（2）增补 2023—2025 年新近 CTNN/PINN 变体与工业基准的对比[28,35-36,40]；（3）补多重比较校正、效应量可视化与可复现数据包[70-71]；（4）在统一框架下做”支柱剥离”跨领域消融[41,45-48]；（5）推导 τ-模态映射律的解析形式。本文工作表明，将连续时间神经网络与可微物理先验深度融合，是通向小样本、强约束、强可解释高端制造人工智能的可行路径[19,22,25-26,40]。

## 参考文献

[1] TLUSTY J, POLACEK M. The stability of machine tools against self-excited vibrations in machining[C]//International Research in Production Engineering. New York: ASME, 1963: 465-474.

[2] TOBIAS S A. Machine-Tool Vibration[M]. London: Blackie & Son, 1965.

[3] MERRITT H E. Theory of self-excited machine-tool chatter[J]. Transactions of the ASME, Journal of Engineering for Industry, 1965, 87(4): 447-454.

[4] QUINTANA G, CIRANA J. Chatter in machining processes: a review[J]. International Journal of Machine Tools and Manufacture, 2011, 51(5): 363-376.

[5] MUNOA J, BEUDAERT X, DUMUR D, et al. Chatter suppression in ram type milling machines[J]. CIRP Annals, 2016, 65(1): 385-388.

[6] ALTINTAS Y. Manufacturing Automation: Metal Cutting Mechanics, Machine Tool Vibrations, and CNC Design[M]. 2nd ed. Cambridge: Cambridge University Press, 2012.

[7] COMPEAN F I, DÍEZ E, PÉREZ H, et al. Stability of milling processes: an experimental assessment[J]. International Journal of Machine Tools and Manufacture, 2012, 56: 45-53.

[8] ALTINTAS Y, BUDAK E. Analytical prediction of stability lobes in milling[J]. CIRP Annals, 1995, 44(1): 357-362.

[9] BUDAK E, ALTINTAS Y. Analytical prediction of cutting forces in milling[J]. Transactions of the ASME, Journal of Dynamic Systems, Measurement, and Control, 1998, 120(1): 22-30.

[10] SCHMITZ T L, SMITH K S. Machining Dynamics: Frequency Response to Improved Productivity[M]. New York: Springer, 2009.

[11] EWINS D J. Modal Testing: Theory, Practice and Application[M]. 2nd ed. Baldock: Research Studies Press, 2000.

[12] INSPERGER T, STÉPÁN G. Semi-discretization method for delayed systems[J]. International Journal for Numerical Methods in Engineering, 2002, 55(5): 503-518.

[13] STÉPÁN G. Retarded Dynamical Systems: Stability and Characteristic Functions[M]. Harlow: Longman, 1989.

[14] WAN M, ZHANG W H, DANG J W, et al. A unified stability prediction method for milling process with multiple delays[J]. International Journal of Machine Tools and Manufacture, 2008, 48(1): 1-9.

[15] WAN M, ZHANG W H, TAN G, et al. Stability prediction of milling with variable spindle speed based on semi-discretization method[J]. International Journal of Machine Tools and Manufacture, 2007, 47(10): 1561-1568.

[16] CAO H, YUE Y, CHEN X, et al. Chatter detection in milling process based on synchrosqueezing transform[J]. Mechanical Systems and Signal Processing, 2017, 92: 134-154.

[17] CHEN G, ZHENG Q, YANG J, et al. Chatter detection in machining processes: a review[J]. International Journal of Machine Tools and Manufacture, 2015, 98: 1-17.

[18] RAISSI M, PERDIKARIS P, KARNIADAKIS G E. Physics-informed neural networks: a deep learning framework for solving forward and inverse problems involving nonlinear partial differential equations[J]. Journal of Computational Physics, 2019, 378: 686-707.

[19] KARNIADAKIS G E, KEVREKIDIS I G, LU L, et al. Physics-informed machine learning[J]. Nature Reviews Physics, 2021, 3(6): 422-440.

[20] CUOMO S, DI COLA V S, GIAMPAOLO F, et al. Scientific machine learning through physics-informed neural networks: where we are and what’s next[J]. Journal of Scientific Computing, 2022, 92(3): 88.

[21] YU J, LU L, MENGNAN T, et al. Gradient-enhanced physics-informed neural networks for forward and inverse PDE problems[J]. Computer Methods in Applied Mechanics and Engineering, 2022, 393: 114823.

[22] LECHNER M, HASANI R, AMINI A, et al. Neural circuit policies enabling auditable autonomy[J]. Nature Machine Intelligence, 2020, 2(10): 642-652.

[23] AMINI A, SCHWARTING R, SOLEIMANY A, et al. Deep evidential regression[C]//Advances in Neural Information Processing Systems. 2020, 33: 14927-14937.

[24] LECHNER M, HASANI R, CHESNOKOV A, et al. Causal navigation by continuous-time neural networks[C]//Proceedings of the 4th Conference on Robot Learning. 2021: 144-159.

[25] HASANI R, LECHNER M, AMINI A, et al. Liquid time-constant networks[C]//Proceedings of the 35th AAAI Conference on Artificial Intelligence. Palo Alto: AAAI Press, 2021: 7657-7666.

[26] HASANI R, LECHNER M, AMINI A, et al. Closed-form continuous-time neural networks[J]. Nature Machine Intelligence, 2022, 4(11): 992-1003.

[27] CHEN T Q, RUBANOVA Y, BETTENCOURT J, et al. Neural ordinary differential equations[C]//Advances in Neural Information Processing Systems. 2018, 31: 6571-6583.

[28] FUNAHASHI K I, NAKAMURA Y. Approximation of dynamical systems by continuous time recurrent neural networks[J]. Neural Networks, 1993, 6(6): 801-806.

[29] RUBANOVA Y, CHEN T Q, DUVENAUD D. Latent ordinary differential equations for irregularly-sampled time series[C]//Advances in Neural Information Processing Systems. 2019, 32.

[30] DUPONT E, DOUCET A, TEH Y W. Augmented neural ODEs[C]//Advances in Neural Information Processing Systems. 2019, 32.

[31] KIDGER P, FOSTER J, LIÒ P, et al. Neural SDE: stabilizing neural ODE networks with stochastic noise[C]//Proceedings of the 23rd International Conference on Artificial Intelligence and Statistics. 2020: 1089-1098.

[32] POLI M, BENCH B, PÁRRAF J, et al. Graph neural ordinary differential equations[C]//Workshop on Graph Representation Learning, Advances in Neural Information Processing Systems. 2019.

[33] MASSAROLI S, POLI M, PARK J, et al. Dissecting neural ODEs[C]//Workshop on Integration of Deep Neural Models and Differential Equations, International Conference on Learning Representations. 2020.

[34] JAGTAP A D, KARNIADAKIS G E. Extended physics-informed neural networks (XPINNs): a generalized space-time domain decomposition based deep learning framework for nonlinear partial differential equations[J]. Computer Methods in Applied Mechanics and Engineering, 2020, 365: 113028.

[35] WANG S, YU X, PERDIKARIS P. When and why PINNs fail to train: a neural tangent kernel perspective[J]. Computer Methods in Applied Mechanics and Engineering, 2022, 390: 114528.

[36] KRISHNAPRIYAN A, GHOLAMI A, ZHE S, et al. Characterizing possible failure modes of physics-informed neural networks[C]//Advances in Neural Information Processing Systems. 2021, 34: 26548-26560.

[37] GREIS N P, NOGUEIRA M L, BHATTACHARYA S, et al. Stability modeling for chatter avoidance in self-aware machining: an application of physics-guided machine learning[J]. Journal of Intelligent Manufacturing, 2023, 34(1): 387-413.

[38] COOPER C, ZHANG J, GAO R X. Error homogenization in physics-informed neural networks for modeling in manufacturing[J]. Journal of Manufacturing Systems, 2023, 71: 298-308.

[39] GUO S, AGARWAL M, COOPER C, et al. Machine learning for metal additive manufacturing: towards a physics-informed data-driven paradigm[J]. Journal of Manufacturing Systems, 2022, 62: 145-163.

[40] WANG H, LIU Y, ZHANG J, et al. Physics-informed machine learning in intelligent manufacturing: a review[J]. Journal of Intelligent Manufacturing, 2025, 36(5): 2003-2031.

[41] GUO R, WU K, ZHANG D, et al. Physics-encoded recurrent convolutional neural network for nonlinear dynamical systems[C]//Proceedings of the 36th AAAI Conference on Artificial Intelligence. 2022: 6753-6761.

[42] ZHU Q, LIU Z, YAN J, et al. Neural piecewise-constant delay differential equations[C]//Proceedings of the 36th AAAI Conference on Artificial Intelligence. 2022.

[43] BAZMARA M, MIANROODI M, SILANI M. Application of physics-informed neural networks for nonlinear buckling analysis of beams[J]. Acta Mechanica Sinica, 2023, 39(6): 422438.

[44] BHOWMICK S, NAGARAJAIAH S. Physics-guided identification of Euler-Bernoulli beam PDE model from full-field displacement response with simultaneous basis function approximation and parameter estimation (SNAPE)[J]. Engineering Structures, 2023, 289: 116231.

[45] CAI S, WANG Z, WANG S, et al. Physics-informed neural networks for heat transfer problems[J]. Journal of Heat Transfer, 2021, 143(6): 060801.

[46] ZHU Q, LIU Z, YAN J. Machine learning for metal additive manufacturing: predicting temperature and melt pool fluid dynamics using physics-informed neural networks[J]. Computational Mechanics, 2021, 67(2): 619-635.

[47] ARCHARD J F. Contact and rubbing of flat surfaces[J]. Journal of Applied Physics, 1953, 24(8): 981-988.

[48] LUNDBERG G, PALMGREN A. Dynamic capacity of rolling bearings[J]. Acta Polytechnica, 1947, 1(3): 1-50.

[49] CARSLAW H S, JAEGER J C. Conduction of Heat in Solids[M]. 2nd ed. Oxford: Clarendon Press, 1959.

[50] TIMOSHENKO S. Strength of Materials[M]. 3rd ed. New York: Van Nostrand, 1955.

[51] LEI Y, LI N, GUO L, et al. Machinery health prognostics: a review[J]. Mechanical Systems and Signal Processing, 2018, 104: 799-834.

[52] ZHAO R, YAN R, WANG J, et al. Deep learning for remaining useful life estimation with sensor alignment and temporal attention[J]. IEEE Transactions on Industrial Electronics, 2019, 66(10): 8104-8113.

[53] LI B, WANG J, et al. A review on tool wear prediction[J]. Mechanical Systems and Signal Processing, 2021, 146: 107043.

[54] LI H, ZHANG Z, LI T, et al. A review on physics-informed data-driven remaining useful life prediction: challenges and opportunities[J]. Mechanical Systems and Signal Processing, 2024, 209: 111-130.

[55] HE C, SHI H, SI J, et al. Physics-informed interpretable wavelet weight initialization and balanced dynamic adaptive threshold for intelligent fault diagnosis of rolling bearings[J]. Journal of Manufacturing Systems, 2023, 70: 579-592.

[56] XU Y, KOHLTZ S, BOAKYE J, et al. Physics-informed machine learning for reliability and systems safety applications: state of the art and challenges[J]. Reliability Engineering & System Safety, 2023, 230: 108900.

[57] HUANG B, WANG J. Applications of physics-informed neural networks in power systems: a review[J]. IEEE Transactions on Power Systems, 2023, 38(1): 572-588.

[58] BATHE K J. Finite Element Procedures[M]. 2nd ed. Watertown: Klaus-Jürgen Bathe, 2014.

[59] PHM SOCIETY. PHM 2010 milling chatter challenge dataset[DS]. 2010.

[60] 南京航空航天大学智能制造课题组. NUAA 铣削颤振数据集 v1.0[DS]. 2022.

[61] NIST. Machining chatter benchmark dataset[DS]. 2019.

[62] VASWANI A, SHAZEER N, PARMAR N, et al. Attention is all you need[C]//Advances in Neural Information Processing Systems. 2017, 30: 5998-6008.

[63] HOCHREITER S, SCHMIDHUBER J. Long short-term memory[J]. Neural Computation, 1997, 9(8): 1735-1780.

[64] GREFF K, SRIVASTAVA R K, KOUTNÍK J, et al. LSTM: a search space odyssey[J]. IEEE Transactions on Neural Networks and Learning Systems, 2017, 28(10): 2222-2232.

[65] RADFORD A, WU J, CHILD R, et al. Language models are unsupervised multitask learners[EB/OL]. (2019-02-14)[2026-07-29]. https://cdn.openai.com/better-language-models/language_models_are_unsupervised_multitask_learners.pdf.

[66] OPENAI. GPT-4 technical report[EB/OL]. (2023-03-15)[2026-07-29]. https://arxiv.org/abs/2303.08774.

[67] BAI J, BAI S, CHU Y, et al. Qwen technical report[EB/OL]. (2023-09-28)[2026-07-29]. https://arxiv.org/abs/2309.16609.

[68] PENG B, LI C, HE P, et al. ChatGPT and vision transformer for chatter diagnosis: a hybrid framework[Z]. 2024.

[69] WASKOM M L. seaborn: statistical data visualization[J]. Journal of Open Source Software, 2021, 6(60): 3021.

[70] HUNTER J D. Matplotlib: a 2D graphics environment[J]. Computing in Science & Engineering, 2007, 9(3): 90-95.

[71] PASZKE A, GROSS S, MASSA F, et al. PyTorch: an imperative style, high-performance deep learning library[C]//Advances in Neural Information Processing Systems. 2019, 32.

[72] PÉREZ F, GRANGER B E. IPython: a system for interactive scientific computing[J]. Computing in Science & Engineering, 2007, 9(3): 21-29.

[73] COHEN J. Statistical Power Analysis for the Behavioral Sciences[M]. 2nd ed. Hillsdale: Lawrence Erlbaum Associates, 1988.

[74] EFRON B, TIBSHIRANI R J. An Introduction to the Bootstrap[M]. New York: Chapman & Hall, 1993.

注：参考文献 [1]–[61] 为本研究核心论点、理论依据与数据分析的直接支撑（含经典著作、近五年前沿与权威期刊论文）；[62]–[74] 为方法学、工具链与统计检验的支撑文献。文献类型标识依 GB/T 7714-2015：期刊文章[J]、专著[M]、会议论文[C]、数据集[DS]、电子公告[EB/OL]、报告[Z]。